F30	ACCESSORIES / SUNDRY ITEMS FOR BRICK / BLOCK / STONE WALLING

To be read with Preliminaries / General conditions.

FLEXIBLE DAMP PROOF COURSES / CAVITY TRAYS

F30/345	SITE FORMED FLEXIBLE SHEET CAVITY TRAYS

- Material : Polymeric

- Manufacturer : RIW Limited, Arc House, Terrace Road South, Binfield, RG42 4PZ.

Tel : 01344  397777  Fax : 01344  862010

E-mail : technical@riw.co.uk   Web : www.riw.co.uk

-	Product reference : RIW Sheetseal 9000 DPC

INSTALLATION OF DPCS / CAVITY TRAYS

F30/415	HORIZONTAL DPC’s

- Placement : In continuous lengths on full even bed of fresh mortar, with 100mm laps at joints and full laps at angles.

- Width : At least full width of leaf unless otherwise specified.

Edges of dpc not covered with mortar.

- Overlying construction : Immediately cover with full even bed of mortar to receive next masonry course.

- Overall finished joint thickness : As close to normal as practicable.

F30/425	GROUND LEVEL DPC’s

- Joint with damp proof membrane : Continuous and effectively sealed.

F30/435	STEPPED DPC’s IN EXTERNAL WALLS

- External walls on sloping ground : Install dpc’s not less than 150mm above adjoining finished ground level.

F30/475 A	SITE FORMED CAVITY TRAYS

- Requirements to prevent downward ingress of water:

-	Profiles : To match those shown on drawings. Firmly secured.

-	Joint treatment : Use unjointed wherever possible, otherwise lap at least 100mm  and seal to provide a free draining and watertight installation.

-    Overlaps and junctions : Seal with RIW Jointing Tape.

-	Horizontal cavity trays : Support using cavity closer.

-	Sloping cavity trays : Prevent sagging.

-	Cleanliness : Free from debris and mortar droppings.